LOCAL STANDARD OPERATING PROCEDURE

JW Marriott Marquis Hotel Shanghai Pudong

LSOP No	:     RM-HSKP-PA-004          	Effective Date   :    Jan 01, 2019

LSOP 编号     :     RM-HSKP-PA-004           生效日期          :    2019年1月1日

Division	:     Rooms                               Revised Date  	  :

部门                :     房务部                              修改日期          :

Section            :     Housekeeping     		Prepared By      :     Director of Service

分部门            :     客房	                         起草人             :     服务总监

Approved By  :      General Manager

批准                :     总经理

Subject	:    CARPET EXTRACTION

标题             ：  地毯去污

OBJECTIVE 目的:

All carpets will be kept clean without stains and damages at all times. Proper cleaning schedule should be followed at all times.

通过合理的计划使酒店内的地毯得到科学的保养,无污渍无破损

POLICY 政策规定:

Let employee know the job announcements, and require the employee to operate strictly according to the requirements

让员工了解工作注意事项，要求员工严格按照要求操作

PROCEDURES 程序:

Use machine to clean carpet

使用机器去清洁地毯

- Remove dust by vacuuming the area thoroughly.

先将所要清洁区域彻底吸尘,去除上面污渍

- Fill the Diluted Cleaner with warm water (1:20) in a pressure pot and sprinkle it on the carpet evenly. Let the cleaner remain on the carpet and let it take effect for a while.

用温水以1:20稀释倒入压力喷壶中,均匀喷在地毯表面.让其作用一会儿.

- Fill the clean water tank with water on the extracting machine. Start cleaning from the inside area with both hands controlling the handle and walking backwards.

在抽洗机水箱中装满清水,推到清洗区域的最里端,启动机器,双手抓住机器手柄向后倒退行走.

- Ensure every line is overlapped by the second one to avoid omitting any area (if necessary, extract a few times to remove the cleaner properly)

吸完一行后,再吸第二行,每一次都要与前一次重叠,避免遗漏. (必要时反复几次,确保清洁剂全部吸走)

- Use the vacuum nozzle to clean the corners.

边角处要使用地毯吸水扒.

- In order to prevent too much foam, foam solution should be put into the dirty water tank when extracting the carpet.

在污水箱中倒入适量消泡剂,防止泡沫过多.

- Use discarded linen to cover the cleaned area after deep cleaning, and remove the water on the skirting board and furniture.

清洁完毕后,用报废床单将所清洁区域铺好,以免踩脏.擦干净地角线及家具上的水渍.

- Clean all tools and machines and take them back to the storeroom.

清洁所有的机器,去除滚刷上的地毯毛等污渍.放回 库房.

- Report any problem if any during carpet extraction to Shift leader as soon as possible.

及时向PA领班报告工作中出现的问题

Carpet cleaning with shampoo machine.

地毯香波清洗地毯

- Vacuum thoroughly the areas which need shampooing.

当需要香波清洗时对地毯进行彻底的吸尘

- Remove all spots in advance

首先去除所有的污迹。

- Prepare all tools: single scrub/form tank/brushes/dry foam cleaner/vacuum

准备好所有工具：单速机/水桶/刷子/干泡沫清洁剂/吸尘器

- Dilute concentrate dry foam cleaner according to situation

根据情况稀释浓缩干泡沫清洁剂。

- Ensure that machine switch is in the off position before plugging into the power supply. The cord should always be behind you

保证在插头插好前机器处在关的状态。绳子应该始终在你身后。

- Ensure that your hands are dry when you turn on the machine

当你开机器时保证你的手是干的

- Move the machine to the inside of cleaning area. Fix brush and switch it on.

把机器移到清洁区域内。安装刷子然后开机器

- Adjust the operation handle.

调整操作把手

- Adjust the quantity of foam according to the situation.

根据现场情况调节出泡量

- Shampoo carpet from left right. The nearest two equal lines must be overlapped, avoid somewhere omitted

从左到右进行香波清洗。最接近的两条清洗线必须交迭，避免遗漏某处。

- When finished shampooing, cover the carpet with discarded linen to avoid being dirty when it is wet. (use air dryer if necessary)

当结束香波清洗，用脏布草覆盖住地毯为了避免它湿的时候变脏。（必要时用通风机）

- When the carpet is thoroughly dry, vacuum the carpet with an upright vacuum.

当地毯彻底干了后，用直立式吸尘器对地毯吸尘

- Carpets should not be shampooed frequently; rather spot leaning is recommended. Inspect the carpets daily.

地毯不能频繁地进行香波清洗；推荐进行污点清洗。每天检查地毯

- If any problem occurs when operating, report to PA shift leader as soon as possible.

当操作时一旦发生任何问题，尽快向PA领班报告

Small area of carpet decontamination

小面积地毯去污

- Vacuum carpet carefully from inside towards the outside. Remove spots and marks immediately when found.

由内向外仔细吸尘。发现污渍及时去除。

- Distinguish the types of stains; use the correct chemicals to remove stain (water/oil-soluble).

鉴别污渍种类。(水溶性或油溶性)。选择正确的清洁剂。

- Dilute shampoo with warm water. Test on a small unnoticeable place to see if it fades.

用温水配好地毯清洁剂。先在一块不被人注意的地方做试验。(检查是否褪色)。

- Apply the dilute shampoo on the stain. (If the stain is solid, scratch it before applying shampoo)

将清洁剂均匀涂在污渍表面。(附着性污渍先用地毯铲刀去除)。

- Remove the stains from outside to inside (avoid spreading)

由外向内刷洗污渍(避免扩散)

- Dry and clean the stained areas with clean dry damp. (Use the mini extractor if necessary)

去除后用干净的湿抹布去除残留的地毯清洁剂(必要时使用小型抽洗机过水)

- After shampooing, the color of carpet should be the same with the original color.

注意去渍后的地毯应与原来的地毯颜色相近。

RESOURCES 相关资料: