LOCAL STANDARD OPERATING PROCEDURE

JW Marriott Marquis Hotel Shanghai Pudong

LSOP No	:     RM-HSKP-PA-006          	Effective Date   :    Jan 01, 2019

LSOP 编号     :     RM-HSKP-PA-006           生效日期          :    2019年1月1日

Division	:     Rooms                               Revised Date  	  :

部门                :     房务部                              修改日期          :

Section            :     Housekeeping     		Prepared By      :     Director of Service

分部门            :     客房	                         起草人             :     服务总监

Approved By  :      General Manager

批准                :     总经理

Subject	:    CHANDELIER CLEANING

标题             ：  吊灯清洁

OBJECTIVE 目的:

Chandeliers should be clean and shining at all times.

使酒店里的吊灯保持高标准的清洁水准

POLICY 政策规定:

Let employee know the job announcements, and require the employee to operate strictly according to the requirements

让员工了解工作注意事项，要求员工严格按照要求操作

PROCEDURES 程序:

Spraying

喷洗

- Switch off the power.

关闭电源

- Lay the appropriate discarded linen under the chandelier.

在吊灯下面铺好报废布草。

- Dilute the glass cleaner (ammonia inclusive) at 1:20. Keep the diluted cleaner in a sprinkling pot.

稀释玻璃清洁剂（有氨） 1：20倒入喷壶中

- Spray the cleaner from top to bottom of the chandelier, standing on a safety ladder.

架好梯子（固定好）由上至下将清洁剂喷在吊灯表面

- Let it take effect for a while, then spray it again with clean water to wash away the remaining cleaner, and then wipe it dry.

待反应一会后在喷壶中加入清水，冲洗吊灯将吊灯表面的清洁剂冲洗干净。用干抹布

擦干吊灯上的水。

Cleaning

彻底清洗

- Prepare all cleaning materials and equipment-bucket, glass cleaner, ladder, and a clean rug.

准备清洗吊灯所需的工具－水桶、玻璃清洁剂、梯子、干净抹布

- Fix the ladder, and remove the chains of the crystal chandelier one by one.

架好梯子，摘下水晶灯链

- Dilute the cleaner in the bucket, and put the chains into the cleaner solution.

在水桶中稀释玻璃清洁剂，将灯链泡在其中

- Wash chains and the chandelier with clean water again and dry it. Then return the chains properly.

洗净灯链并擦干，用干抹擦干灯具。将灯链按原位挂好。

Note： 注意事项

- when using a ladder, safety- precautions must be followed, i.e. assistance to hold the ladder etc.

当使用梯子时，一定要注意安全。例如：有人在梯子旁守护。

- all cleaning operation high more than 1 meter, it should work by night shift of the outsourcing company, hotel staff can’t do the high-altitude cleaning operations

所有一米高空清洁作业都由夜班外包公司负责，酒店员工不得私自进行高空清洁作业。

RESOURCES 相关资料: