LOCAL STANDARD OPERATING PROCEDURE

JW Marriott Marquis Hotel Shanghai Pudong

LSOP No	:     RM-HSKP-PA-009          	Effective Date   :    Jan 01, 2019

LSOP 编号     :     RM-HSKP-PA-009           生效日期          :    2019年1月1日

Division	:     Rooms                               Revised Date  	  :

部门                :     房务部                              修改日期          :

Section            :     Housekeeping     		Prepared By      :     Director of Service

分部门            :     客房	                         起草人             :     服务总监

Approved By  :      General Manager

批准                :     总经理

Subject	:    MARBLE MAINTENANCE

标题             ：  大理石保养

OBJECTIVE 目的:

Marbles will be crystallized with the right method to ensure clean and shining marble at all times.

使酒店的大理石得到合理的保养，保持其清洁光亮。

POLICY 政策规定:

Let employee know the job announcements, and require the employee to operate strictly according to the requirements

让员工了解工作注意事项，要求员工严格按照要求操作

PROCEDURES 程序:

Marble Crystallization

大理石结晶

- Prepare all necessary accessories.

准备清洁所需的全部工具。

- Single Scrub Machine         b)  Wet/dry vacuum cleaner          c)  Needle Base

单擦机                                          吸水机                                             针盘

d)   Red Mat              	              e)   Measure cup                             f) Bucket

红垫                                	        量杯                                                 水桶

g)   Notice sign               	  h)   Polishing machine

告示牌                                          抛光机

- Place notice signs around cleaning area.

在清洁区域周围设置告示牌。

- Wash the cleaning area thoroughly with single scrub machine and purpose cleaner.

先将清洁区域彻底洗净。

- Compound the crystal power 1:1 in measure glass with water. Apply on the marble evenly. NOTE: The area should be around 3-5m2 at a time.

用量杯以1:1配制结晶粉。均匀地涂在理石上。注意每次面积不要太大(3-5M2)

- Fix needle base and Red Mat on the single scrub machine. Turn on the machine, rubbing the areas applied the compound until the marble become shinning.

使用单擦机。安装针盘、红片。在涂好结晶剂的区域研磨，直至地面光亮。

- If the corner area is encountered, it can be polished by hand throwing machine (especially small areas can be polished by hand red Mat)

如遇到边角区域可使用手抛机进行打磨（特别小的区域使用手工红片打磨）

- After crystallization, wash the area again with single scrub machine and absorb the dirty water with wet vacuum cleaner

结晶完毕后，用吸水机将地面吸净，并用尘推将地坪推干净

- Clean all accessories and take them back to the storeroom.

清洗全部的工具，存放在库房。

Daily cleaning.

日常清洁

- Mop every day to remove any stains discovered.

每天推尘，发现污渍及时去处

- Mop should be pushed in a straight line, do not omit any area.

推尘需以直线的推，避免遗漏

Daily Polish

日常抛光

Prepare all necessary accessories.

- .准备清洁所需的全部工具。

- Crystal  Machine                            b)  Wet/dry vacuum cleaner          c)  Needle Base

晶面机                                                   吸尘器                                          针盘

d)   steel wool （0#）               e)   watering can                               f) dust

钢丝棉（0#）                                   喷壶                                     尘推

g)   Notice sign

告示牌

- Place notice signs around cleaning area

在清洁区域周围设置告示牌

- Wash the cleaning area thoroughly with single scrub machine and purpose cleaner.

先将清洁区域彻底洗净

- Wire wool is wound in a circular clockwise direction on a needle disk and installed on a planar machine

将钢丝棉以圆形顺时针缠绕在针盘上安装在晶面机上

- Spray the marble polish evenly on the marble to polish and clean the stone surface

将云石抛光剂均匀的喷洒在大理石上，进行石材表面的抛光清洁

- NOTE: The area should be around 3-5m2 at a time.

注意每次面积不要太大(3-5M2)

- Use Crystal machine and marble polish chemical thoroughly clean and polish the marble.

使用晶面机和云石抛光剂进行石材表面的抛光清洁。

- In case of corner area, it can be polished by hand throwing machine (especially small area, it can be polished by hand steel wire cotton).

如遇到边角区域可使用手抛机进行打磨（特别小的区域使用手工钢丝棉打磨）

- After polishing, dust and vacuum cleaning

抛光后, 用尘推及吸尘器清洁.

- Polish the floor during no guest time. If there is a guest passing by during the polish process, stop and greeting the guest with smile.

在没有客人时，对地面进行抛光。如在抛光过程中有客人经过，必须停下工作并向客人微笑问好。

RESOURCES 相关资料: